New Christie Mirage SST-6P RGB pure laser projection system redefines the 3D experience

CYPRESS, Calif. – (March 23, 2021) – Christie® today announced the launch of the new Christie Mirage SST-6P, a dual-head 18,000 lumen, 4K, 6-Primary 3D RGB pure laser projection system that delivers twice the onscreen brightness of traditional color-comb 30,000-lumen lamp-based 3D projection systems for bright, colorful, and detailed 3D images for venues including theme parks, attractions and giant screen dome theaters.

The 6-Primary (6P) 3D dual-projector system uses two sets of RGB laser primaries with slightly off-set wavelengths: one for the right eye, and one for the left. Reusable passive 3D glasses filter different wavelengths and direct the light to the correct eye, with no shuttering, for a natural and comfortable 3D visual experience. The result is a projection system that is 90% efficient at projecting light onto the screen, and white-screen compatible for blending the multi-projector arrays typically used in dome theaters and media-based attractions such as dark rides and flying theaters.

“With the Mirage SST-6P projection system, dark 3D visuals are a thing of the past,” says Larry Paul, executive director, Technology and Custom Solutions, Enterprise and Entertainment, Christie. “The Mirage SST-6P was developed specifically for 3D applications and exceeds the DCI-P3 color space, ensuring that 3D visuals in dark rides, attractions and planetariums look as intended – bright, contrast-rich and colorful – revealing a level of detail audiences haven’t seen before.”  With TruLife™ electronics, Mirage SST-6P delivers 4K 3D content up to 120fps per eye, with an option of up to 480fps at 2K.

The projection system uses two fiber-coupled projection heads with remote light sources and chillers. The projection heads are quiet, compact and lightweight, and have an extremely low heat output of less than 2400 BTU each. With omni-directional capabilities and solid-state illumination, Mirage SST-6P is designed for installation flexibility in tight, challenging environments and 30,000 hours of virtually maintenance-free operation to 80% brightness.

Christie Mirage SST-6P includes Christie Twist™, for built-in warping and blending. It is compatible with Christie Mystique™ , an automated camera-based alignment software solution, and supports Christie Guardian for invisible, real-time, automatic blended-image correction, even when content is playing.

Christie Mirage SST-6P is available for order now.

About Christie®

Christie Digital Systems USA, Inc. is a global visual and audio technologies company and a wholly owned subsidiary of Ushio Inc., Japan (JP: 6925). Christie revolutionized the movie industry with the launch of digital cinema projection, and since 1929 has embraced innovation and broken many technology barriers. From the world’s largest mega-events to the smallest applications, our technology—from advanced RGB pure laser and SDVoE technology to image processing and LED display solutions—empowers people to create the world’s best shared experiences. Visit www.christiedigital.com.

– 30 –

For more information contact:

Deborah Noon

Christie

(519) 749-3109

deborah.noon@christiedigital.com

Follow Christie:

https://www.twitter.com/christiedigital

https://www.twitter.com/christievive

https://www.facebook.com/christiedigital

https://www.linkedin.com/company/christie-digital-systems

https://www.youtube.com/christiedigital

https://www.instagram.com/christiedigital

https://vimeo.com/christiedigital

“Christie” is a trademark of Christie Digital Systems USA, Inc., registered in the United States of America and certain other countries.

“DLP” is a registered trademark of Texas Instruments